GrabServer2

A Program to Acquire Images from Cameras

### Programmer's Documentation

by Stefan Weiße

sweisse@ifh.de

##### March 28, 2003

Table of Contents

1. Introduction	3

2. Overview of the Classes	4

2.1. Class Chart	5

2.2. Description of the classes	6

2.3. Application Flow	10

3. Selected Documentation of Details	11

3.1. Protocol of the Socket Connection	11

3.2. TINE Frame Transfer	13

3.3. TINE Properties	14

3.4. Description of the Configuration File	18

4. Compile Guide	20

# 1. Introduction

This program is meant to acquire images from cameras that are driven in triggered acquisition mode. The frames that will be acquired are compressed and send out to connected clients either via TCP/IP socket connections or via TINE (Three-fold Integrated Network Environment) protocol in multicast operation mode. In addition to the video frame data connections, which are only used for transmitting the video data, there are also control connections using the TINE protocol. These control connections are used for remotely switching the camera port and to transfer camera port description strings, scale values and other important data from server to client.

Key features of the server:

- multi-threaded

- externally triggered acquisition of images

- loss-less compression

- switching of the camera port (even while continuous grabbing)

- configurable camera description strings and scale values for the 4 camera ports

- spot size and centre calculation on selected camera using Fourier analysis

- socket module to send out the video data to each connected client via TCP/IP streaming sockets

- TINE multicast module to send out the video data using multicast operation mode

- rotation and flipping of video frames on server side in order to fit the client side orientation

- driven by configuration file

The Server is written in C++ language, by making use of the integrated development environment Microsoft Visual C++ 6. Inline assembler is used in some speed critical parts of the program. The source code is about 9200 lines long (303 KB) including comments. It is very well inline commented.

Information on how to set up and start the server is collated in the Quick Setup Guide. This documentation is meant only for programmers, who want to understand how the software works.

# 2. Overview of the Classes

The program is constructed out of 17 classes. Most classes are helper classes for the main class of the application, which is the CMainDlg class. The CMainDlg class is the class behind the grabber server's main dialog and manages the whole program.

#### Class	Description

CAboutDlg	about dialog of the application

CBogusTineServerDlg	sends out frames using TINE multicast

CCompression	compresses the frames using a loss-less Huffman based compression

CConfigDlg	configuration dialog of the application

CFourierStat	spot centre and size detection based on Fourier analysis of the video data

CGrab	encapsulation of the raw ITEX grabbing functions

CGrabsrvApp	application class, just instantiates the main dialog

CIniEx	class that manages the configuration file handling

CMainDlg	main dialog class, manages the whole application

CMemoryManagement	encapsulation of memory allocation and memory copy

COrientationHelper	rotates or flips the video frame to fit client-side orientation

CPtrQueue	a pointer queue for transmitting frames across thread boundaries

CSendReceive	manages sending and receiving on the socket connections

CSocketEncapsulation	encapsulates everything connected to sending of frames

CSrv	wrapper class of the TINE ActiveX server control

CStat	base class for spot centre and size detection, needed by CFourierStat

SSyncSocket	fast synchronous socket class

## 2.1. Class Chart

## 2.2. Description of the classes

CAboutDlg		about dialog of the application

This is the MFC class for the About dialog of the application. It is connected tightly to the resource of the about dialog.

This class is used in CMainDlg class.

#### CBogusTineServerDlg		sends out frames using TINE multicast

Using the bogus TINE server dialog, the application is able to send out video frames using TINE multicast operation mode. The to-be-send-out frames are put into the class by CSocketEncapsulation class. If there is a request from client side, the frame gets send out. This class is used inside CSocketEncapsulation class.

CCompression		loss-less compression of frames using Huffman based codec

This class encapsulates all functionality that is needed to access the HuffYUV codec for loss-less compression of the video frames. The codec is accessed by making use of the VCM (Video Compression Manager) function, which are built into Windows. The only use of this class is to be able to compress the frames that are acquired by the frame grabber.

This class is used in CMainDlg class.

CConfigDlg		configuration dialog of the application

This is the MFC class for the configuration dialog. It is connected tightly to the resource of the configuration dialog. By using the dialog the user is able to change the preferences of the application. The required preferences are loaded from and saved to a configuration file by use of the CIniEx class. There are some functions that get activated when the user makes certain operations on the dialog's controls ('On…'-functions).

Using the configuration dialog, one can set the ITEX system number and board number, the listen port and the maximum connections on the sockets architecture as well as the camera description strings, scale values and orientation of each camera port.

This class is used in CMainDlg class.

CFourierStat		spot centre and size calculation based on Fourier analysis

This class inherits the CStat class. It replaces the spot centre and size calculation . These values are calculated based on Fourier analysis. This class is needed for proper calculation of spot centre and size. Some client can obtain these values. A laser control system is planned. This will obtain these values by making use of the TINE protocol and adjusting the mirrors according to the calculated values.

This class is used in CMainDlg class.

CGrab		encapsulation of the raw ITEX grabbing functions

This class encapsulates the low-level ITEX functions, which are needed for properly accessing the frame grabber card. The class drives the frame grabber card in triggered acquisition mode, acquires the frames in a thread and posts a message to the main dialog indicating that the frame was acquired and is ready to be fetched from the grab buffer. The main application class then fetches the frame, compresses it and puts it into the CSocketEncapsulation class, which is responsible for properly sending out the frame.

Beside the grabbing, this class is also responsible for switching of the camera port by making use of some raw ITEX functions.

This class is used in CMainDlg class.

CGrabsrvApp		application class, just instantiates the main dialog

This is the main application class. The execution path starts here. It is responsible for properly starting up the main application dialog (CMainDlg). All functionality is then handled there.

#### CIniEx		class that manages the configuration file handling

This class is needed for initialisation file handling. It is an external class, which was downloaded from the web. One can load and save an initialisation file and access the members of the file through a Section/Key/Value schema. For an overview on the settings that are located in the configuration file, one should take a look at chapter 3.3 .

This class is used in CMainDlg class and in CConfigDlg class.

CMainDlg		main dialog class, manages the whole application

This is the main class of the application. It holds the main message queue. All other classes are just driven by this main class. There are a lot of  'On…' functions which gets activated by events like pressing a button etc. It also has a lot of 'Initialize…' and 'Deinitialize…' functions. Every component of the program is initialised at start up and deinitialised  at shut down of the program.

This class is used in CGrabsrvApp class.

CMemoryManagement 	encapsulation of memory allocation and memory copy

The CMemoryManagement class is a wrapper class for aligned memory allocation and memcpy(). One can easily add different implementations of aligned memory allocation and memory copy routine(s).

This class is global to the whole application.

COrientationHelper		rotates or flips the video frame to fit client-side orientation

By using this class, the application is able to apply certain rotations and flipping operations on the source video frame that is acquired by the frame grabber. It is needed because the video frame should always fit to the client-side orientation of the coordinate system. This class is used in CMainDlg class.

CPtrQueue		a pointer queue for transmitting frames across thread boundaries

This class was written to transport grabbed frames across thread boundaries. The frame (pointer) is input on one side, and on the other side the pointer can be read out of the queue. The whole class is thread-safe at object level. This is achieved by using the MFC synchronisation classes CSingleLock and CCriticalSection.

This class is needed for interaction between CSendReceive class and the send and receive threads. In CSendReceive (main thread) a frame is put into the queue. In the CSendReceive send thread the frame is get out of the queue and send out using SSyncSocket class.

This class is used in CSendReceive class.

CSendReceive		manages sending and receiving on the socket connections

This class manages sending and receiving on one of the sockets a client is connected to. For simultaneously transmitting and receiving, the class starts up two threads. The send thread gets a frame out of the attached queue and sends it out. The receive thread does not receive anything, it is just there to check whether the connection has been closed by the client.

This class is used in CSocketEncapsulation class.

CSocketEncapsulation		encapsulates everything connected to sockets

This class handles the whole video frame transfer based part of the application. It is responsible for accepting incoming connections and for sending out the data properly. For incoming socket connections, this class runs a listen thread, which listens for incoming connections and accepts them. The class also checks whether the connection is closed. If this happens, it shuts down the send and receive threads (CSendReceive) and cleans up everything on the socket. From application point of view, one just has to initialise this class (Init() ) and puts data (video frames) into it. The rest is handled by the class itself. For TINE multicast mode, the class instantiates CBogusTineServerDlg in order to send out the frames using TINE.

This class is used in CMainDlg class.

CSrv		wrapper class of the TINE ActiveX server control

This class was entirely generated by Visual C++. It is a wrapper class in order to properly access the 'srv' ActiveX control, which is a TINE server. Using this wrapper class the control connections, like changing the camera port and sending out the camera strings to connected clients are implemented. In addition, the sending out of frame using TINE is also implemented using CSrv object.

This class is used in CMainDlg class.

CStat		base class for spot centre and size detection,

needed by CFourierStat

This class is the base class for statistical analysis of the video data. It features spot centre and size calculation based on 'Weight of the Masses' algorithm. In this project, the class is just needed as a base class for CFourierStat.

SSyncSocket			fast synchronous socket class

The socket class is needed because the normal MFC-based CSocket class is rather slow. It encapsulates the raw Windows socket API. It is synchronous, which means that each operation, which can not be processed immediately, blocks the execution flow. Because of this, most of the functions are only called from threads, in order not to block the main execution flow.

This class is used in CSendReceive class and in CSocketEncapsulation class.

## 2.3. Application Flow

When the grabber server is started up, the configuration settings are read in from the configuration file. After reading in the configuration file, the application is initialised based on the parameters from the configuration file. This all happens in OnInitDialog() of CMainDlg class. In addition, some memory is allocated for properly storing application specific data. If every step of the initialisation succeeded some buttons on the main dialog get activated. If some initialisation fails, one can only close the application or start up the configuration dialog to enter the proper settings.

It is assumed that the initialisation succeeded. The grabbing can be started up by pressing on the 'Start Grabbing' button. When this button is pressed, the OnButtonStartGrabbing() function in CMainDlg gets called. There the grabbing is started (by calling StartGrab() function of CGrab class).

After starting the grabbing, the function OnNotifyGrabbed() is called periodically when a new frame is acquired by the ITEX frame grabbing API. By getting the frame out of the grab queue, the actual frame bits as well as some information on the frame are transferred. After getting the data, the OnNotifyGrabbed() function rotates or mirrors the frame to fit the client side orientation. The information how the image should be rotated or flipped can be set up using the configuration dialog. After applying the desired orientation, a spot centre and size analysis using the CFourierStat class is done. The returned spot centre and size can be obtained by reading out a TINE property. After applying the analysis, the function OnNotifyGrabbed() tries to compress the frame using CCompression class. If the compressing is successful (means that the compressed frame is smaller than an uncompressed frame) the compressed frame gets send out. Otherwise it is replaced by an uncompressed frame. The selected frame is passed to the CSocketEncapsulation class, which is responsible for properly sending out the frame to all connected clients. In addition, some controls on the main dialog (framerate, heart beat) are updated for every new frame that comes in.

Inside CSocketEncapsulation class, the frame is put into every client's CSendReceive class. This class duplicates the frame by making a local copy. This copy is then send out using the send thread of each CSendReceive class. For TINE protocol support, the frame is put into CBogusTineServerDlg class by the CSocketEncapsulation class. Inside this bogus dialog class the sending out of the frames using TINE protocol is handled.

This flow is repeated for every new frame that comes in. The whole process of sending out the frames is decoupled. This means that if one client blocks and is not able to receive any more frames, the other clients still get their frames in time.

# 3. Selected Documentation of Details

## 3.1. Protocol of the Socket Connection

The protocol between server and client is pretty simple. After the client successfully connects to the server, the server starts to send one video frame after another. The server receives nothing through the socket connection.

A full frame is made out of two or three parts. It depends whether the frame is compressed or not. At first, the video header is send out for each frame. This structure is 88 bytes long and contains several important information concerning the frame. For example, the frame number, a timestamp, a compression flag, length of the frame bits and other useful information is transmitted.

After the video header, the raw data follows. When a frame is send out compressed, a second header (format header) is transmitted after the video header. This header is needed by the video CODEC for properly decompressing the video frame. Because the decompression routines are built into the client, this header is not used. For compatibility reasons, this header is still transmitted. Later, it could be that this header is needed to support different compression methods. Also some old versions of the video client rely on this format header.

After the format header, the frame bits follow. The size of the frame bits is encoded in the video header (VIDHDR.len_data).

If a frame is send out uncompressed, the format header is omitted. Just after the video header the uncompressed frame bits follow. The size of the uncompressed frame bits is also encoded in the video header (VIDHDR.len_data).

An algorithm to receive the frames through a socket connection would be:

- read in the video header (88 bytes)

- is the frame compressed ? (VIDHDR.compressed == 1)

- if yes, read in format header (length: VIDHDR.len_fmthdr)

- if yes, read in compressed frame bits (length: VIDHDR.len_data)

- if not, read in uncompressed frame bits (length: VIDHDR.len_data)

- repeat from the beginning

When the compressed frame bits are read into a memory location, one should add at least 8 bytes for safety. This is needed for properly decompressing the frame bits.

Looking at the source, the structure of the video header has the following design:

typedef struct {

UINT   kennung;      // 0x11223344;

UINT   compression;  // FOURCC codec name

UINT   len_total;    // length of data + formatheader + vidhdr

UINT   len_data;     // length of the data (compressed image)

UINT   len_fmthdr;   // length of the format header (needed for

// decompression on windows side)

UINT   frameno;      // since starting the grabserver

_timeb timestamp;    // time in milliseconds since 1970 // _ftime() _timeb

// structure

INT    cameraport;   // port of the camera (0..3)

UINT   widthframe;   // width of the frame in pixels

UINT   heightframe;  // height of the frame in pixels

UINT   compressed;   // 1= picture is compressed

// 0= picture is not compressed because if it would be

// compressed it would be too big

double framerate;    // floating point: average number of frames per second

double scale;        // scale value of the video frame (pixel-mm ratio)

UINT   bpp;          // bits per monochrome pixel (e.g. 8)

char   reserved[12]; // reserved for future enhancements

} VIDHDR;

All data is transmitted in Little Endian format (Intel PC). If the video header is read on a different platform (e.g. SUN (Big Endian)), the values have to be converted accordingly.

In the following table the structure members are explained.

Name 			Description

kennung		Identification (marker) of the video header, contains 0x11223344

compression		FOURCC (Four Character Code) of the codec that was used

The content would be MAKEFOURCC('H','F','Y','U').

len_total	total length of the video frame (video header + (format header) + frame bits) in bytes

len_data		length of the frame bits in bytes

len_fmthdr	length of the format header (for a compressed frame) in bytes, for an uncompressed frame this member variable should be zero

frameno		number of the frame (incremented by 1 for each new frame)

timestamp		timestamp (the time the frame was acquired by the framegrabber card)

cameraport		the camera port the frame was taken from (0..3)

widthframe	width of the video frame in pixels

heightframe 	height of the video frame in pixels

compressed		flag that indicates whether a frame is compressed or not

1 = frame was compressed; 0 = frame was not compressed

framerate		floating-point variable with the actual framerate (approximately)

scale	floating-point variable containing the scale of the video frame (pixel to millimetre ratio)

bpp			bits per pixel (e.g. 8)

reserved		12 reserved bytes for future enhancements

## 3.2. TINE Frame Transfer

Beside the socket connection, there exists a second way to acquire the video frames from the server. This option uses the TINE protocol in multicast operation mode. Using the multicast mode, server resources are saved and the required network bandwidth is lower. It does not matter anymore how many clients are connected, the amount of data that is required to be send out is the same no matter 1 or 10 clients are connected. The network hardware is responsible to pass the video frames to all connected clients.

When the video frames are transferred using TINE protocol,  the used data transfer mode is polling (POLL.NETWORK) with a repetition rate of 20 Hz. 20 times per second (one request every 50 milliseconds) the client should ask the server whether a new frame is ready to be send out. If there is a new frame, it gets send out. The client then receives the frame.

For querying the server for a video frame, there exists a special property.

Device context: "PITZ"

Device server	: hostname of server in upper case plus an additional 'F'

Location	: "device_0"

Property	: "FRAME.GET"

The device server string depends on the hostname of the grabber server computer. If the hostname is for example znppitzfg1, the device server name would be "ZNPPITZFG1F". For the second installed grabber server (znppitzfg2) it would be "ZNPPITZFG2F".

When the client receives an answer from the grabber server when requesting to read the FRAME.GET property, there are two possible status codes the server can reply. The first one is 0, which means that the frame was transmitted by the server and is delivered together with the status code. If the status code is 607 it means that the server has no new video frame available and the client should repeat to read the property until the status code is zero. If there is any other status code than 0 or 607 received by the client, one should read the TINE manual what the status code means, because it is a pure TINE error and has nothing to do with the frame transfer over an established TINE link.

When receiving the video frame through the TINE connection, the client program receives an array of varying length, which contains all information necessary. The array consists of the video header, an optional format header and the frame bits, either compressed or uncompressed. The video header, the optional format header and the frame bits are stored one after another.

## 3.3. TINE Properties

Apart from the video frame connections, the server also offers some TINE properties. Reading and writing these properties from any client application running on a Windows PC or Unix workstation that has the TINE protocol installed offers many additional features of the program.

Using these properties one can:

- change the camera port

- read out the actually selected camera port

- get the camera description strings

- get actual spot centre and size values

- read out width and height of the desired camera port

- read out scale value of the desired camera port

- read out bits per pixel setting of the server

- read out the FOURCC code of the used video codec

To access a certain property one has to know four settings (device context, device server, device name and device property):

Device Context : PITZ

Device Server: (same name as the computer name of the server in upper case, e.g. ZNPPITZFG1)

Device Name: "device_0"

Device Property: the desired property

For the parameter "Device Property" , 21 different strings can be specified:

1.	"GRAB_CAM" 		- Property to retrieve/select the camera port (0..3)

2.	"GRAB_CAM.0"		- Property to retrieve description string of camera port 0

"GRAB_CAM.1"		- Property to retrieve description string of camera port 1

"GRAB_CAM.2"		- Property to retrieve description string of camera port 2

"GRAB_CAM.3"		- Property to retrieve description string of camera port 3

3.	"GRAB_CAM.AKT"		- Read out the description string of the actual camera port

4.	"GRAB_CAM.SPOTDATA"	- Acquire the actual spot size and center position

5.	"SCALE.0"			- Read out scale value of camera port 0

"SCALE.1"			- Read out scale value of camera port 1

"SCALE.2"			- Read out scale value of camera port 2

"SCALE.3"			- Read out scale value of camera port 3

6.	"GRAB_WIDTH.0"		- Read out width (in pixels) of camera port 0

"GRAB_WIDTH.1"		- Read out width (in pixels) of camera port 1

"GRAB_WIDTH.2"		- Read out width (in pixels) of camera port 2

"GRAB_WIDTH.3"		- Read out width (in pixels) of camera port 3

7.	"GRAB_HEIGHT.0"		- Read out height (in pixels) of camera port 0

"GRAB_HEIGHT.1"		- Read out height (in pixels) of camera port 1

"GRAB_HEIGHT.2"		- Read out height (in pixels) of camera port 2

"GRAB_HEIGHT.3"		- Read out height (in pixels) of camera port 3

8.	"GRAB_BPP"			- Read out bits per pixel setting of server

9.	"GRAB_COMPR"		- Read out FOURCC code of video codec used

1.	When using "GRAB_CAM", one has two options. It is possible to either write a single byte identifying the camera port one wants to select or one can read a single byte and get the actually selected camera port back. The single byte can have integer values of 0 up to 3. 4 different camera ports can be selected. When a single byte is read out, the TINE server sends the actually selected camera port back. The byte that is returned can have integer values of 0 to 3.

In addition to the actual data, TINE also sends some status code back. The status code for the remote call can be one of the following:

EqpStatus Code	Description

0			everything okay, either camera port is set or properly sent to client 602			could not change the camera port

603			wrong camera port value (not in 0..3)

601	could not read property (no byte sent identifying the camera port one wants to select to when writing the property)

>0 && <=512		internal TINE error

If one wants to set the camera port using TINE, a big timeout value (e.g. 4 seconds) is recommended because switching the camera port may take a while.

2. 	Using the properties "GRAB_CAM.x"  (0 <= x <= 3) , one is able to obtain the four description strings of the camera ports. The x stands for the camera port (0..3) the description string is wanted from. The principle of the call is the same for all four properties. One has to read a NAME48 (TINE data type) string from the server. In this string, the camera description string of the camera port is returned.

Beside the NAME48 string, the server sends some status code back, which can be one of the following:

EqpStatus Code	Description

0			everything okay, string is returned

607			could not send the string to the client

>0 && <=512		internal TINE error

If the error code 0 is returned, the NAME48 string contains the camera port description string of the desired camera port.

3.	The property "GRAB_CAM.AKT" (read camera description string of actual camera port) is analogical to the GRAB_CAM.x property, one just gets the string of the actually selected camera port back.

4.	Using the property "GRAB_G_SPOTDATA", one is able to obtain the actual position and size of the spot that is visible on the video frame.

After the RPC call, which is specified by the 4 parameters (Context/Server/Device/ Property), a data packet (16 bytes) is sent back to the client. The data type is IFFF (Integer-Float-Float-Float, also called Filter). This data format is used for compatibility reasons to DOOCS data types.

The 16 byte packet is build up the following way:

Position	Length		Declaration	Description

0		4 Bytes	int xm;		X-position of the spot centre (in pixels)

4		4 Bytes	float ym;	Y-position of the spot centre (in pixels)

8		4 Bytes	float xrms;	Size of the spot in X-direction (in pixels)

12		4 Bytes	float yrms;	Size of the spot in Y-direction (in pixels)

To transport the floating point variable xm through an integer, the value was multiplied by 1000 before it was converted to integer. To get the real value back, one should convert it back to floating point and divide it by 1000.

The calculated coordinates depend on the width and height of the selected camera port. As video frames can be rotated, the lower right corner x and y position can change. The x position of the lower right corner equals always to the width of the video frame minus one. The y position of the lower right corner equals always to the height of the video frame minus one.

For example, the width of the video frame is 768 and the height is 574. Inside the video frame the upper left corner in pixel-coordinates (x,y) would be (0,0), the lower right corner in pixel coordinates (x,y) would be (767,573). If a spot has coordinates of xm=50 and ym=550. The spot would be located in the lower left corner of the screen.

The interval for the X variables is [0,width-1] and [0,height-1] for Y.

Aside from the returned data the server sends a status code back, which can be one of the following:

EqpStatus Code	Description

0			everything okay, spot data structure is returned

601			internal error, no spot data is returned

606                              spot data could not be sent out

>0 && <=512		internal TINE error

5.	Using the property "SCALE.x" (0 <= x <= 3) one can read out the scale value of

the desired camera port. The scale value (floating-point) is returned as double datatype.

Besides the scale value there is a status code transferred, which can be one of the following values:

EqpStatus Code	Description

0			everything okay, scale value is returned

607			error while transferring the scale value back to client

>0 && <=512		internal TINE error

6.	Using the property "GRAB_WIDTH.x" (0 <= x <= 3) one can read out the width (in pixels) of the desired camera port. The width is transferred as 32-bit unsigned integer (4 bytes).

Besides the width value there is a status code transferred, which can be one of the following values:

EqpStatus Code	Description

0			everything okay, width is returned

607			error while transmitting width back to client

>0 && <=512		internal TINE error

7.	Using the property "GRAB_HEIGHT.x" (0 <= x <= 3) one can read out the height (in pixels) of the desired camera port. The height is transferred as 32-bit unsigned integer (4 bytes).

Besides the height value there is a status code transferred, which can be one of the following values:

EqpStatus Code	Description

0			everything okay, height is returned

607			error while transmitting height back to client

>0 && <=512		internal TINE error

8.	Using the property "GRAB_BPP" one can read out the bits per pixel setting of the server. The bits per pixel setting is transferred as byte datatype (1 byte).

Besides the bits per pixel value there is a status code transferred, which can be one of the following values:

EqpStatus Code	Description

0			everything okay, bits per pixel setting is returned

607			error while transmitting bits per pixel setting back to client

>0 && <=512		internal TINE error

9.	Using the property "GRAB_COMPR" one can read out the four character code (FOURCC) of the video codec used for compressing the video frames. The FOURCC code is transferred as 32-bit unsigned integer (4 bytes).

Besides the FOURCC code, there is a status code transferred, which can have one of the following values:

EqpStatus Code	Description

0			everything okay, FOURCC code is returned

607			error while transmitting FOURCC code back to client

>0 && <=512		internal TINE error

There is one more parameter than the ones that are described. This is the 'GRAB_CAM_R' property. It is a deprecated property which is just left in for compatibility reasons.

Everything connected to TINE inside the GrabServer2 is handled in three special procedures of CMainDlg class. These are 'InitializeTine()', 'DeinitializeTine()' and 'OnEqpFcnSrvctrl1()'.

## 3.4. Description of the Configuration File

To remember the settings, the server creates a configuration file on first start-up. On next start-ups, the server will read the configuration file in. It also modifies the configuration file on exit. The name of the file is 'Grabsrv2.cfg'.

A typical configuration file would be:

--- snip ---

[DEFAULT]

BOARDNUMBER=0

SYSTEMNUMBER=0

MAXCLIENTS=255

LISTENPORTNUMBER=31777

LASTPORT=0

NEGATIVEREFERENCE=0

POSITIVEREFERENCE=1000

CAMERAPORT0STRING=Virtual Cathode

CAMERAPORT1STRING=EMSY

CAMERAPORT2STRING=Dispersive Arm

CAMERAPORT3STRING=NOT Connected

CAMERAPORT0SCALE=0.008300

CAMERAPORT1SCALE=0.025670

CAMERAPORT2SCALE=0.028329

CAMERAPORT3SCALE=1.000000

CAMERAPORT0ORIENTATION=MIRRORHORIZ

CAMERAPORT1ORIENTATION=ROT90CWMIRRHORIZ

CAMERAPORT2ORIENTATION=MIRRORVERT

CAMERAPORT3ORIENTATION=NORM

--- snap ---

The file is made out of one section ('DEFAULT') that contains key-value pairs. The order of the lines does not matter. The BOARDNUMBER and SYSTEMNUMBER keys are the PCVision settings. As in the configuration dialog, the board number can range from 0..9 and the system number can range from 0..7. Both values have the type integer.

The MAXCLIENTS is the network setting 'max. Clients' on the configuration dialog. The values must be integer. The normal range would be 5..255.

The LISTENPORTNUMBER is the port number the server listens for incoming client connections. The value is of type integer and can range between 0..65535. The value should be left on default of 31777.

The setting 'LASTPORT' is updated any time the application is exit. The last active camera port setting is remembered. In this example, the last active camera port was 0. This setting should not be modified by hand. It is of type integer and the range is between 0 and 3.

The two settings positive reference and negative reference are needed for setting the threshold of the ADC inside the frame grabber card. These values are calculated through a calibration process and should not be changed ! The range of the negative reference is from 0 to 1200. The range of the positive reference is from 0 to 2000. The unit of both reference values is millivolts. Adding up both values the result must not exceed 2000. The following formula illustrates this:

p.ref. + n.ref. <= 2000

The four CAMERAPORTxSTRING (0 <= x <= 3) are the four camera port description strings. The value has the type string. A camera port description string should not contain more than 32 characters.

Using the four CAMERAPORTxSCALE (0 <= x <= 3) one can set up the scale value (ratio of pixels to millimetres) of the desired camera port. It is a floating-point value. Using this value the pixel values are translated to millimetre values on client side.

The CAMERAPORTxORIENTATION (0 <= x <= 3) describe how the image should be rotated or mirrored before it is analysed and sent out to the connected clients. It is needed to fit the orientation of the client side programs. The following values can be used:

Value				Description

NORM			do nothing, leave the image as it is

ROT90CW			rotate the image by 90 degrees clockwise

ROT180CW			rotate the image by 180 degrees

ROT270CW			rotate the image by 270 degrees clockwise

MIRRORHORIZ		mirror the image along the horizontal axis

MIRRORVERT		mirror the image along the vertical axis

ROT90CWMIRRHORIZ 	rotate the image by 90 degrees clockwise and mirror the image along the horizontal axis afterwards

# 4. Compile Guide

In this section everything is explained concerning the compilation of the GrabServer2 project. In order to properly compile the grabber server the following is needed:

- 	Microsoft Visual C++ 6.0 SP5

-	Microsoft Processor Performance Pack for Visual C++ 6 (vcpp5.exe)

installed into Visual C++

-	ITEX 4.1 include directory

-	TINE include and library directory

"\\mkinthera1\hera\kernel\win32.gui\server"

-	TINE ActiveX control srv.ocx properly installed on the  PC

Inside the project, there is a configuration for the Intel C++ Compiler. If the Intel compiler instead of the Microsoft Compiler built into Visual C++ is used, a significant speed improvement can be gained. One has to choose 'Release Intel' as the configuration, enable the Intel compiler and build the project.

When the workspace file ('Grabsrv.dsw') is opened the first time in Visual C++, one should open

Project->Settings and edit the two additional include directories. It is necessary that the proper ITEX 4.1 include directory, as well as to the proper TINE include directory are given. After this step was repeated for all configurations, one should check whether the TINE ActiveX control "srv" is installed properly. One should open the resources and select the main dialog

(Dialog->IDD_GRABSRV_DIALOG). It is installed properly if no error message pops up that the ActiveX control could not be found. If the error message pops up maybe the control is just not registered properly. Normally it resides in

"<WINDOWS HOME DIR>\system32\". The filename is "srv.ocx". To register it one has to  execute "regsvr32.exe srv.ocx" on the command line.

The next step is to adjust the TINE library directory. At least the linking process needs the file 'Winsrv32.lib'. Under normal circumstances it is located in the same directory as the include files. The directory is "\\mkinthera1\hera\kernel\win32.gui\server".

Once the include and library path are set up, the next step would be to choose the proper configuration and try to rebuild it (using Build->Rebuild All). Compiling the sources may take a while. If there was an error, one has probably not properly set up the include directories. It should be double checked whether the directories and network shares exist. There might be errors if the Intel configurations are compiled with the Microsoft compiler or vice versa.