Can Parameters:

int cdiAddr;       /* module address */

int cdiSubAddr;    /* module sub-address */

cdiAddr === > Can Node Id:

cdiSubAddr = not used, set to 0

enum execCanParamIndex

{

canOpenId, /* can open id */

canOpenObj, /* can open object number */

canOpenSubIndex,

canBroadcast, /* canID forced to be 0, one telegram for all devices */

canAutoReceive, /* device sends data regulary (for Read or ReadthenWrite) */

};

canOpenId:

PDO_TRANSMIT(0x200), PDO2_TRANSMIT(0x300). NMT_COMMANDS(0x0), SDO_TRANSMIT(0x600), PDO_DI_READ(0x180), PDO_AI_READ(0x280)

canOpenObj: only for SDO transmit

canOpenSubIndex:

Can Node Id = 5

PDO DI receive, sent by device regularly

Then input:

5.0:0x180:0:0:0:1

#define NMT_START_REMOTE 0x1 /* NMT data byte */

#define NMT_STOP_REMOTE 0x2

#define NMT_ENTER_PREOPERATION 0x80

#define NMT_RESET_NODE 0x81

#define NMT_RESET_COMMUNICATION 0x82

#<busname>:<parameters>,<line>,<index>,<address>.<subaddress>:<parameter1>:<parameter2:<etc>,,,,<device number>,<device name>

BUS,LINE,INDEX,ADDRESS,MASK,PATTERN,TOLERANCE,NUMBER,NAME

CANESD:500,1,0,3.0:0:0:0:1:0,0xffff,0,0,22,ESDCAN_INIT

CANESD:500,1,1,3.0:0x600:0x1000:0:0:0,0xffff,0,0,23,ESDCAN_READ1

Cdimf.csv file:

BUS, BUS ID, LIBRARY

CANESD,3, canEsd.dll

SEDPC,1, cdiSedPc.dll

WINCOMM,2, cdiWinComm.dll

canMsgTypeIndex:

0:   sdoMsg,

1:   pdo1Msg,

2:   pdo2Msg,

3:   nmtMsg,

4:   emergencyMsg,

5:   notdefinedMsg,

execCanParamIndex

{

msgType,   /* canMsgTypeIndex:   0, 1, 2, 3, 4, 5(any message) */

canOpenObj, /* can open object number */

canOpenSubIndex,

canBroadcast, /* canID forced to be 0, one telegram for all devices */

canAutoReceive, /* device sends data regulary (for Read or ReadthenWrite) */

}

Can Node Id = 5

PDO DI (0x180) receive, sent by device regularly

Then input:

5.0:1:0:0:0:1

Can Node Id = 20

SDO Index: 0x3000, Sub Index: 0x10

Then input:

20.0:0:0x3000:0x10:0:0